2: Construction science principles

# Worksheet 14: Stress and strain (tutor)

Engineers use various calculations to establish whether a designed structure is safe. Some of these calculations are used to determine:

- stress – calculated using force/area

- strain – calculated by the variation in length/original length, and represented by a ratio.

1. A force of 450kN is exerted over a square concrete column of side length 300mm. Calculate the stress within the concrete.

The force is 450kN

The area is 300 x 300mm

450/(300 x 300) = 0.005 kN/mm2

2. The allowable stress within a steel beam carrying a load of 300N is 1.33N/mm2. Calculate the cross-sectional area of the steel beam in mm2.

1.33 = 300 / X

X = 300 / 1.33 = 225 mm2

3. A column is axially loaded, and contracts 0.15mm. The length of the column is 3500mm. Calculate the strain.

The length is 3500mm

The variation in length is 0.15mm

Variation in length/original length = strain

0.15/3500 = 0.000042