2: Construction science principles

# Worksheet 18: U-values (tutor)

A U-value is a measure of heat loss/gain found within building materials used for walls, floors and roofs. U-values are always measured in W/m2K (watts per metre squared kelvin).

The higher the U-value of a material, the worse it performs in relation to retaining heat. So materials with a low U-value have high levels of heat retention.

Calculate the U-value of a wall consisting of the following elements:

- RSi = 0.125

- 100mm blockwork = thermal conductivity (K) = 0.84

- 100mm cavity = R = 0.1

- 100mm aircrete blocks K = 0.95

- 100mm external insulation R =2.0845

- 15mm render K = 0.45

0.125 + 0.1/0.84 + 0.1 + 0.1/0.95 + 2.0845 + 0.015/0.45 + 0.055

1/Rt = U-value

Summate and take reciprocal:

= 1/2.62

= 0.38

Learners could be asked to look at the building regulations Part L on insulation to find out what a typical value should be for an external wall.

For more support in calculating U-values, visit:

https://www.archdaily.com/898843/how-to-calculate-the-thermal-transmittance-u-value-in-the-envelope-of-a-building#:~:text=How%20to%20Calculate%20the%20Thermal%20Transmittance%20%28U-Value%29%20in,transmittance%20specified%20for%20the%20climatic%20zone%20in%20