2. Construction science principles

# Worksheet 3: Materials tests (tutor)

All the tests in this worksheet can either be tutor demonstrations or class activities, at the tutor’s discretion and depending on time and facilities available.

In order for materials to meet their required standards, testing has to be undertaken to check that a supplied material complies with its specification.

At your tutor’s direction, undertake one or all of the following simple tests and record the results, following your tutor’s instructions.

- A silt test for a mortar sand using the following link and equipment supplied.

- https://theconstructor.org/building/building-material/test-silt-content-sand/34520/

Record your results in the table. If you have time to carry out more than one test on the sand, you will obtain a more accurate result based on average readings.

|  | Description | Sample 1 | Sample 2 | Sample 3 |
| --- | --- | --- | --- | --- |
| 1 | Volume of sample sand (V2) |  |  |  |
| 2 | Volume of silt layer (V1) |  |  |  |
| 3 | Percentage of silt (Calculate this as follows: (V1/V2) x 100 = % of silt) |  |  |  |
|  | Average silt content |  |  |  |

Ask students to read the test instructions online. You may wish to run through them with learners before embarking on the test. Learners may need to be supervised to carry out the test in pairs or groups. The test should be carried out at the start of the class to allow sufficient time to measure the results at the end of the session.

- A slump test on a concrete mix using the following link and equipment supplied.

Watch the video at: https://www.youtube.com/watch?v=yzpWGrh9j6Y. Now follow you tutor’s instructions to carry out the slump test. Observe and measure the results of the slump test. What did you notice?  Did the concrete mix collapse, shear or slump? What did this tell you about the mix?

Tutor will need to borrow a slump test from a contractor, and or hire one in.

Tutor should learners to watch the video first.

Learners should identify the difference between a collapse, shear and slump. They should also understand that a collapse or shear demonstrates an inappropriate concrete mix.

- Carry out a brick absorption test.

  - Dry out some bricks in an oven

  - Weigh each brick and number it

  - Half submerge each brick in water for one hour

  - Weigh each brick

  - Calculate the % of water absorbed

An oven with different brick samples will be required for the absorption test.