3. Construction design principles

# Worksheet 14: Planning permission (1) (tutor)

Circle the correct answers to these questions.

- What can be built under permitted development rights?

- A new two-storey house in an existing garden.

- A new single-storey building in its own plot.

- A two-storey extension to a single-storey house.

- A single-storey extension to a two-storey house.

- Is building control required when building using permitted development rights?

- Yes, because all new builds require final approval.

- No, as visits are only required during demolition.

- No, as its only required under retrospective planning.

- Yes, as approval is only required for small projects.

- What are the purposes of submitting an outline planning application? Select two answers.

- To see if full approval might be granted in principle.

- It confirms that all building regulations are complied with.

- To show the local planning department detailed drawings.

- Removes the need for the submission of full planning permission.

- Full detailed drawings and specification are not required at this stage.

- What does the term ‘planning breach’ mean?

- Construction has started without any planning approval in place.

- Construction has started after planning approval has been granted.

- Construction has been paused due to amendments to the drawings.

- Construction has been paused due to amendments to the building regulations.

- When would retrospective planning permission be required?

- When building a detached garage on client’s own land.

- When work has started without any planning permissions.

- When converting a shop into multi-residential accommodation.

- When the proposed development has a footprint greater than 200m2.